SPECIFICATION

Sections 07 90 00 / 07 95 00

DFR-FP by EMSEAL

Watertight, Fire Rated Split Slab/Plaza-Deck Expansion Joint System
Preformed, Pre-Compressed, Self-Expanding, Fire Rated Sealant System with Silicone Pre-Coated Surface, Watertight, Traffic Durable, 2 or 3 Hour Rated, Primary Seal for Horizontal Joints

***Note to Specifier:

It is critical in ensuring the success of the project that this expansion joint be specified in the same section as the deck waterproofing system and is installed by the contractor responsible for installing the deck waterproofing system.

***

# PART 1 – GENERAL

  - Work Included

- The work shall consist of furnishing and installing waterproof expansion joints in accordance with the details shown on the plans and the requirements of the specifications. Expansion joints shall be silicone-coated, precompressed foam seal installed into metal rail system leveled and bolted to the structural slab and incorporating integral side flashing sheets for watertight, sandwich encapsulation into the split-slab waterproofing membrane by others.

- Related Work

- Division 3 - Section - Cast-in-Place Concrete

- Division 7 - Thermal & Moisture Protection

- Division 7 - Sealants, Caulking and Waterproofing

- Division 7 – Joint Firestopping

  - Submittals

- General – Submit the following according to Division 1 Specification Section.

- Standard Submittal Package – Submit typical expansion joint drawing(s) indicating pertinent dimensions, general construction, expansion joint opening dimensions and product information.

- Alloys where used:

- Plates: Aluminum 6061-T6 alloy meeting ASTM B221-95a

- Plates: Stainless Steel Grade 304 meeting ASTM A240 and A666

- Capping Strip: Stainless Steel Grade 304 meeting ASTM A240 and A666

- Legs: Commercial  Grade Galvanized Steel meeting ASTM A653

- Sample of material to be used in work is required at time of submittal.

- All products must be identified by a UL listing number and must be listed in the UL and ULC Online Certification Directories as proof that they have been tested according to UL 2079 and manufactured under UL’s “Follow Up” service.

- All products must be certified by independent laboratory test report to be free in composition of any waxes or wax compounds using FTIR and DSC testing.

- All products shall be certified in writing to be: a) capable of withstanding 150°F (65°C) for 3 hours while compressed down to the minimum of movement capability dimension of the basis of design product (-25% of nominal material size) without evidence of any bleeding of impregnation medium from the material; and b) that the same material after the heat stability test and after first being cooled to room temperature will subsequently self-expand to the maximum of movement capability dimension of the basis-of-design product (+30% of nominal material size) within 24 hours at room temperature 68°F (20°C).

  - Product Delivery, Storage and Handling

- Deliver products to site in Manufacturer’s original, intact, labeled containers. Handle and protect as necessary to prevent damage or deterioration during shipment, handling and storage. Store in accordance with manufacturer’s installation instructions.

  - Basis of Design

- All joints shall be designed to meet the specified performance criteria of the project as manufactured by: (USA & International) EMSEAL JOINT SYSTEMS, LTD 25 Bridle Lane, Westborough, MA 01581-2603, Toll Free: 800-526-8365. (Canada) EMSEAL, LLC 111 Royal Group Crescent, Woodbridge, ON, L4H 1X9 Canada Toll Free: 800-526-8365.  www.emseal.com

- Alternate manufacturers must demonstrate that their products meet or exceed the design criteria and must submit certified performance test reports performed by nationally recognized independent laboratories as called for in section 1.02 Submittals. Submittal of alternates must be made three weeks prior to bid opening to allow proper evaluation time.

  - Quality Assurance

- The General Contractor will conduct a pre-construction meeting with all parties and trades involved in the treatment of work at and around expansion joints including, but not limited to, concrete, mechanical, electrical, HVAC, landscaping, masonry, curtain wall, waterproofing, fire-stopping, caulking, flooring and other finish trade subcontractors. All superintendents and foremen with responsibility for oversight and setting of the joint gap must attend this meeting.  The General Contractor is responsible to coordinate and schedule all trades and ensure that all subcontractors understand their responsibilities in relation to expansion joints and that their work cannot impede anticipated structural movement at the expansion joints, or compromise the achievement of watertighness or life safety at expansion joints in any way.

- Warranty – Manufacturer’s standard warranty shall apply.

- LEED Building Performance Requirements: The VOC of the silicone must not exceed 50 grams/liter.

## PART 2 – PRODUCT

  - General

- Provide traffic durable, watertight, fire-rated, expansion joint by EMSEAL Joint Systems for expansion joints and isolation joints in plaza decks and split slabs. Typical locations include, but are not limited to the following: applications for joints over occupied space, stadium concourse decks, parking deck joints, plaza and podium decks, airport roadways, mall bridge connectors, between split-slab and solid slab construction, and structural expansion joints. System shall perform waterproofing, fire-rating, traffic bearing and movement-accommodation functions as the result of a single installation and without the addition of fire blankets, fire barriers, mineral wool, coverplates, gutters, vapor barriers, bladders, or other devices suspended beneath or within the system in any way.

- Provide DSM-FP as manufactured by EMSEAL JOINT SYSTEMS LTD and as indicated on drawings for horizontal expansion joint locations.

- Sealant system shall be comprised of two subassemblies: a) the joint sealing assembly and b) the structural-slab mounted supporting legs with integral waterproofing side sheets. The two subassemblies shall be comprised of the following components: 1) fire-retardant-impregnated foam pre-coated on the underside with an intumescent fire-proofing material, 2.) pre-coated on the traffic surface with highway-grade, fuel resistant silicone, 3) field-applied epoxy adhesive primer, 4) field-injected silicone sealant bands.  Impregnation agent to have proven non-migratory characteristics, 5.) structural-slab mounted retainer legs, 6.) integral heat-weldable NBR modified PVC waterproofing side sheets, 7.) stainless steel capping strips, 8.) hi-mod epoxy-gel leveling bed and dielectric separator layer, 9.) carbon Steel Grade II zinc dichromate yellow finish, UNC 16, anchors and nuts and, and 10.) hi-mod anchor epoxy.

- Material shall be capable as a dual seal, of movements of +/-25% (50% total) of nominal material size.  Standard sizes from 1” (25mm) to 4” (150mm).  Depth of seal is 5” (125mm).

- Silicone coating to be highway-grade, low-modulus, jet-fuel resistant silicone factory-applied to the foam while it is partially pre-compressed to a width greater than maximum joint extension and cured before final compression. When compressed to final supplied dimension, a bellow(s) to handle movement must be created in the silicone coating.

- Epoxy-mortar setting/leveling bed to act as dielectric separator as well as to ensure that the system is fully supported and at the appropriate elevation throughout its length.

- Manufacturer’s field-applied epoxy gel adhesive is applied to the faces of the previously installed mounting leg assembly. DFR-FP foam seal is lowered into the adhesive slightly recessed from the surface.  Field-injected silicone sealant bands shall be injected at the bellow to leg assembly interface to complete the waterproofing.

- The side flashing sheets shall be 12” (300 mm) wide and .012” (3mm) thick.

- Side flashing sheets are locked into a reglet in top surface of retainer leg on each side of joint. With the side flashing sheets pulled out of the way deck waterproofing membrane (by others) is installed on the deck and brought over and up the DFR-FP system retainer legs. The side flashing sheets are lowered into the liquid membrane (or into the non-sag mastic component of a sheet waterproofing system by others) and sandwiched with another layer of waterproofing. Paver, asphalt or wearing course material is installed up to the stainless steel retaining caps on the mounting rails. Concrete as a wear course is poured up to a ¼” (6mm) form strip (by others) placed against the stainless steel retainer caps. After the concrete has cured, the form strip is removed and the control joint caulked with a liquid sealant (by others).

- Select the system model  appropriate to the movement and design requirements at each joint location that meet the project specification or as defined by the structural engineer of record.

- Manufacturer’s Checklist must be completed by expansion joint subcontractor and returned to manufacturer at time of ordering material.

  - Fabrication

- DFR-FP by EMSEAL must be supplied precompressed to less than the joint size, packaged in shrink-wrapped lengths (sticks).

- For the mounting rail and side sheet assembly, directional changes and terminations into vertical plane surfaces (walls, parapets, ends of decks, etc) as well as to transition the material through curbs, treads and risers or other in-slab plane changes to be provided by factory-manufactured assemblies to field measurements provided by the expansion joint subcontractor.

- Expansion joint supporting aluminum rail extrusions to be factory set at mid-point of movement range and held at this dimension by spacers to be removed after attachment of the rails to the deck.  Furnish in lengths to minimize number of end joins.

## PART 3 – EXECUTION

  - Installation

- Preparation of the Work Area

- The contractor shall provide a properly formed and prepared expansion joint openings constructed to the exact dimensions and elevations shown on manufacturer’s standard system drawings or as shown on the contract drawings. Deviations from these dimensions will not be allowed without the written consent of the engineer of record.

- The contractor shall clean the joint opening of all contaminants immediately prior to installation of expansion joint system.  Repair spalled, irregular or unsound joint surfaces using accepted industry practices for repair of the substrates in question.  Remove protruding roughness to ensure joint sides are smooth.  Refer to Manufacturers Installation Guide for detailed step-by-step instructions.

- System to be installed by qualified sub-contractors only according to detailed published installation procedures and/or in accordance with job-specific installation instructions of manufacturer’s field technician. Bids must include as a specific line item the presence of paid-for manufacturer’s field technician during initial preparation, inspection, and material installation. The applicator must be the same contractor as will be installing the deck waterproofing system.

  - Clean and Protect

- Protect the system and its components during construction. Subsequent damage to the expansion joint system will be repaired at the general contractor’s expense. After work is complete, clean exposed surfaces with a suitable cleaner that will not harm or attack the finish.

END OF SECTION

Rev. 03-07-2024